Thu Aug 22 17:37:44 PDT 1991

The following are results of tests of different

methods of measuring the soma.

The first line after each run of ntscable shows measurements

in which diameter is that of the circle having the same

2D area as that enclosed by the soma outline (z coordinate

neglected).

The second line shows results based on calculations from

the 3D point list, either with or without using the

z coordinate, as indicated.

The value on the third line is a normalized difference

between the area measurements:

(A2-A1) / (A2+A1)

where A1 is the area from the first line and A2 is area from

the second line.  The means of the absolute values of these

numbers are also shown at the bottom of each group.

using z coordinate:

ntscable -v -n300 -a1 121l02.lgn junk.hoc

soma diam = 22.0251;  length = 22.0251 area = 1524

soma diam = 17.4622;  length = 39.6793 area = 2176.78

0.176

ntscable -v -n300 -a1 121r03.lgn junk.hoc

soma diam = 17.4899;  length = 17.4899 area = 961

soma diam = 11.9263;  length = 24.2308 area = 907.869

-0.028

ntscable -v -n300 -a1 123l06.lgn junk.hoc

soma diam = 17.7428;  length = 17.7428 area = 989

soma diam = 12.1109;  length = 27.6317 area = 1051.32

0.030

shelton % ntscable -v -n300 -a1 123r01.lgn junk.hoc

soma diam = 20.1495;  length = 20.1495 area = 1275.5

soma diam = 11.8343;  length = 33.5217 area = 1246.28

-0.0116

ntscable -v -n300 -a1 124l04.lgn junk.hoc

soma diam = 24.6054;  length = 24.6054 area = 1902

soma diam = 15.7833;  length = 38.0991 area = 1889.13

-0.0033

shelton % ntscable -v -n300 -a1 j4.txt junk.hoc

soma diam = 22.3408;  length = 22.3408 area = 1568

soma diam = 15.5067;  length = 28.2618 area = 1376.79

-0.0649

shelton % ntscable -v -n300 -a1 c12* junk.hoc

soma diam = 17.6664;  length = 17.6664 area = 980.5

soma diam = 20.4191;  length = 16.2912 area = 1045.06

0.0321

mean = 0.04957

ignoring z coordinate:

ntscable -v -n300 -a1 121l02.lgn junk.hoc

soma diam = 22.0251;  length = 22.0251 area = 1524

soma diam = 17.0518;  length = 32.0791 area = 1718.47

0.0599

ntscable -v -n300 -a1 121r03.lgn junk.hoc

soma diam = 17.4899;  length = 17.4899 area = 961

soma diam = 12.1178;  length = 21.7137 area = 826.623

-0.075

shelton % ntscable -v -n300 -a1 123l06.lgn junk.hoc

!soma diam = 17.7428;  length = 17.7428 area = 989

soma diam = 12.8476;  length = 23.2325 area = 937.712

-0.0266

ntscable -v -n300 -a1 123r01.lgn junk.hoc

soma diam = 20.1495;  length = 20.1495 area = 1275.5

soma diam = 11.9965;  length = 29.3389 area = 1105.72

-0.071

ntscable -v -n300 -a1 124l04.lgn junk.hoc

soma diam = 24.6054;  length = 24.6054 area = 1902

soma diam = 16.6441;  length = 32.852 area = 1717.8

-0.0509

shelton % ntscable -v -n300 -a1 j4.txt junk.hoc

soma diam = 22.3408;  length = 22.3408 area = 1568

soma diam = 15.4923;  length = 28.1249 area = 1368.85

-0.0678

shelton % ntscable -v -n300 -a1 c12* junk.hoc

soma diam = 17.6664;  length = 17.6664 area = 980.5

soma diam = 19.8998;  length = 14.4836 area = 905.473

-0.0395

mean = 0.05590

nevin syntax (no soma outline):

shelton % ntscable -v -n300 -a1 0422.out junk.hoc

soma diam = 43.68;  length = 70.8579 area = 9723.45

soma diam = 24.1074;  length = 85.7631 area = 6495.31